Table 6.2.6.4.2 Assessable development - Urban neighbourhood precinct

| Performance outcomes | Examples that achieve aspects of the Performance Outcomes | E Compliance - Yes - No See PO or - NA | Justification for compliance |
| --- | --- | --- | --- |
| General criteria |  |  |  |
| Density |  |  |  |
| PO1 The Urban neighbourhood precinct has a medium to high residential density of at least 45 dwellings per ha (site density). | E1 Residential uses have a minimum site density of: - 75 dwellings per ha for sites shown on: - ‘Figure 6.2.6.4.1 - Kallangur’ - Kallangur; - ‘Figure 6.2.6.4.2 - Mango Hill’ - Mango Hill; - ‘Figure 6.2.6.4.3 - Mango Hill East’ - Mango Hill East; - ‘Figure 6.2.6.4.4 - Murrumba Downs’ - Murrumba Downs; - ‘Figure 6.2.6.4.5 Kippa-Ring’ - Kippa-Ring; or - Overlay map - Building heights as having a building height maximum of 27m and a minimum of 8.5m; - 45 dwellings per hectare for all other areas. |  |  |
| Residential uses |  |  |  |
| PO2 Dual Occupancies(21) and low density residential uses are not located in this precinct. | No example provided. |  |  |
| Building height (Residential uses) |  |  |  |
| PO3 Buildings and structures have a height that: - is consistent with the medium to high rise character of the Urban neighbourhood precinct; - responds to the topographic features of the site, including slope and orientation; - is not visually dominant or overbearing with respect to the streetscape; - responds to the height of development on adjoining land where contained within another precinct or zone. Note - Refer to Planning scheme policy - Residential design for details and examples.. | E3 Building height: - is within the minimum and maximum mapped on Overlay map – Building heights; or - for domestic outbuildings, including free standing carports and garages, 4m and a mean height not exceeding 3.5m. |  |  |
| Building height (Non-residential uses) |  |  |  |
| PO4 The height of non-residential buildings does not adversely affect amenity of the area or of adjoining properties. | E4 Building height does not exceed the maximum height identified on Overlay map - Building heights except for architectural features associated with religious expression on Place of worship(60) and Educational establishment(24) buildings. |  |  |
| Setbacks (Residential uses) |  |  |  |
| PO5 Residential buildings and structures are setback to: - be consistent with medium to high density Urban neighbourhood precinct character where buildings are positioned close to the footpath to create active frontages; - maintain private open space areas that are of a size and dimension to be usable and functional; - maintain the privacy of adjoining properties; - ensure parked vehicles do not restrict pedestrian and traffic movement and safety; - limit the length, height and openings of boundary walls to maximise privacy and amenity on adjoining properties; - ensure built to boundary walls do not create unusable or inaccessible spaces and do not negatively impact the streetscape character, amenity or functionality of adjoining properties; - Provide adequate separation to particular infrastructure and water bodies to minimise adverse impacts on people, property, water quality and infrastructure. Note - Refer to Planning scheme policy - Residential design for details and examples.. | E5.1 Setbacks (excluding built to boundary walls) comply with Table 6.2.6.4.3 ‘Setbacks’ - Setback (Residential uses). Note - Greater setbacks may be required if the lot adjoins an environmental corridor or area (Refer to values and constraints for details).. |  |  |
|  | E5.2 Buildings (excluding class 10 buildings and structures) ensure that built to boundary walls are: - of a length and height in Table 6.2.6.4.4 ‘Built to boundary walls (Residential uses)’; - setback from the side boundary: - not more than 20mm; or - if a plan of development shows only one built to boundary wall on the boundary, not more than 150mm; - on the low side of a sloping lot. Editor's note - Lots containing built to boundary walls should also include an appropriate easement to facilitate the maintenance of any wall within 600mm of a boundary. For boundaries with built to boundary walls on adjacent lots a 'High Density Development Easement' is recommended; or for all other built to boundary walls a 'easement for maintenance purposes' is recommended.. |  |  |
| Setbacks (Non-residential uses) |  |  |  |
| PO6 Front setbacks ensure non-residential buildings address and actively interface with streets and public spaces. | E6.1 For the primary street frontage buildings are constructed: - to the property boundary; or - setback a maximum of 3m from the property boundary, where for the purpose of outdoor dining. |  |  |
|  | E6.2 For the secondary frontage, setbacks are consistent with adjoining buildings. |  |  |
| PO7 Side and rear setbacks cater for driveway(s), services, utilities and buffers required to protect the amenity of adjoining sensitive land uses. | No example provided. |  |  |
| Site cover (residential uses) |  |  |  |
| PO8 Residential buildings and structures will ensure that site cover: - does not result in a site density that is inconsistent with the character of the area; - does not result in an over development of the site; - does not result in other elements of the site being compromised (e.g. Setbacks, open space etc); - ensures that buildings and structures reflect the attached medium to high density urban character. Note - Refer to Planning scheme policy - Residential design for details and examples.. | E8 Site cover (excluding eaves, sun shading devices, patios, balconies and other unenclosed structures) does not exceed the specified percentages in the table below. Building height; Lot Size. 300m2 or less; 301- 400m2; 401- 500m2; 501- 1000m2; 1001- 2500m2; Greater than 2501m2. 8.5m or less; 75%; 70%; 60%; 60%; 60%; 60%. >8.5m to 12.0m; 50%; 50%; 60%; 50%; 50%; 50%. >12.0m to 21m; N/A; N/A; 50%; 50%; 40%; 40%. >21m to 27m; N/A; N/A; N/A; N/A; 35%; 35%. Greater than 27m; N/A; N/A; N/A; N/A; 25%; 25%. Note - Refer to Planning scheme policy - Residential design for details and examples.. |  |  |
| Movement network |  |  |  |
| PO9 Development is designed to connect to and form part of the surrounding neighbourhood by providing interconnected street, pedestrian and cyclist pathways to adjoining development, nearby centres, neighbourhood hubs, community facilities, public transport nodes and open space. | E9.1 Development provides and maintains the connections shown on: - ‘Figure 6.2.6.4.6 - Dakabin’ - Dakabin; - ‘Figure 6.2.6.4.7 - Kallangur’ - Kallangur; - ‘Figure 6.2.6.4.8 - Mango Hill’ - Mango Hill; - ‘Figure 6.2.6.4.9 - Mango Hill East’ - Mango Hill East; - ‘Figure 6.2.6.4.10 - Murrumba Downs ’- Murrumba Downs; - ‘Figure 6.2.6.4.11 - Narangba east ’- Narangba; - ‘Figure 6.2.6.4.12 - Petrie’ - Petrie. |  |  |
|  | E9.2 All other areas, no example provided. |  |  |
| Water sensitive urban design |  |  |  |
| PO10 Best practice Water Sensitive Urban Design (WSUD) is incorporated within development sites adjoining street frontages to mitigate impacts of stormwater run-off in accordance with Planning scheme policy - Integrated design. | No example provided. |  |  |
| Setbacks to sensitive land uses |  |  |  |
| PO11 Sensitive land uses within 250m of land in the Industry zone - General industry precinct must mitigate any potential exposure to industrial air, noise or odour emissions that impact on human health, amenity and wellbeing. Note - A noise impact assessment may be required to demonstrate compliance with this PO. Noise impact assessments are to be prepared in accordance with Planning scheme policy – Noise.. | E11 Development is designed and operated to ensure that: - it meets the criteria outlined in the Planning Scheme Policy - Noise; and - the air quality objectives in the Environmental Protection (Air) Policy 2008, are met. |  |  |
| Amenity |  |  |  |
| PO12 The amenity of the area and adjacent sensitive land uses are protected from the impacts of dust, odour, noise, light, chemicals and other environmental nuisances. | No example provided. |  |  |
| Noise |  |  |  |
| PO13 Noise generating uses do not adversely affect existing or potential noise sensitive uses. Note - The use of walls, barriers or fences that are visible from or adjoin a road or public area are not appropriate noise attenuation measures unless adjoining a motorway, arterial road or rail line.. Note - A noise impact assessment may be required to demonstrate compliance with this PO. Noise impact assessments are to be prepared in accordance with Planning scheme policy - Noise.. | No example provided. |  |  |
| PO14 Sensitive land uses are provided with an appropriate acoustic environment within designated external private outdoor living spaces and internal areas while: - contributing to safe and usable public spaces, through maintaining high levels of surveillance of parks, streets and roads that serve active transport purposes (e.g. existing or future pedestrian paths or cycle lanes etc); - maintaining the amenity of the streetscape. Note - A noise impact assessment may be required to demonstrate compliance with this PO. Noise impact assessments are to be prepared in accordance with Planning scheme policy - Noise.. Note - Refer to Planning Scheme Policy – Integrated design for details and examples of noise attenuation structures.. | E14.1 Development is designed to meet the criteria outlined in the Planning Scheme Policy – Noise. |  |  |
|  | E14.2 Noise attenuation structures (e.g. walls, barriers or fences): - are not visible from an adjoining road or public area unless: - adjoining a motorway or rail line; or - adjoining part of an arterial road that does not serve an existing or future active transport purpose (e.g. pedestrian paths or cycle lanes) or where attenuation through building location and materials is not possible. - do not remove existing or prevent future active transport routes or connections to the street network; - are located, constructed and landscaped in accordance with Planning scheme policy - Integrated design. Note - Refer to Planning scheme policy – Integrated design for details and examples of noise attenuation structures.. Note - Refer to Overlay map – Active transport for future active transport routes.. |  |  |
| Clearing of habitat trees where not located within the Environmental areas overlay map |  |  |  |
| PO15 - Development ensures that the biodiversity quality and integrity of habitats is not adversely impacted upon but maintained and protected. - Development does not result in the net loss of fauna habitat. Where development does result in the loss of a habitat tree, development will provide replacement fauna nesting boxes at the following rate of 1 nest box for every hollow removed. Where hollows have not yet formed in trees > 80cm in diameter at 1.3m height, 3 nest boxes are required for every habitat tree removed. - Development does not result in soil erosion or land degradation or leave land exposed for an unreasonable period of time but is rehabilitated in a timely manner Note: Further guidance on habitat trees is provided in Planning scheme policy - Environmental areas. | No example provided. |  |  |

| Works criteria |  |  |  |
| --- | --- | --- | --- |
| Utilities |  |  |  |
| PO16 Where the site adjoins or is opposite to a Park(57), foreshore or Humpybong Reserve all existing overhead power lines are to be undergrounded for the full frontage of the site. | No example provided. |  |  |
| PO17 The development is connected to an existing reticulated electricity supply system approved by the relevant energy regulating authority. | E17 Development is connected to underground electricity. |  |  |
| PO18 The development has access to telecommunications and broadband services in accordance with current standards. | No example provided. |  |  |
| PO19 Where available the development is to safely connect to reticulated gas. | No example provided. |  |  |
| PO20 The development provides for the treatment and disposal of sewage and other waste water in a way that will not cause environmental harm or pose a risk to public health. | E20.1 Where in a sewered area, the development is connected to a reticulated sewerage network. |  |  |
|  | E20.2 Trade waste is pre-treated on-site prior to discharging into the sewerage network. |  |  |
| PO21 The development is provided with an adequate and sustainable supply of potable (drinking and general use e.g. gardening, washing, fire fighting) water. | E21 Where in an existing connections area or a future connections area as detailed in the Unitywater Connections Policy, the development is connected to the reticulated water supply system in accordance with the South East Queensland Water Supply and Sewerage Design and Construction Code and the relevant Water Service Association of Australia (WSAA) codes and standards. |  |  |
| PO22 The development is provided with constructed and dedicated road access. | No example provided. |  |  |
| Access |  |  |  |
| PO23 Where required, access easements contain a driveway and provision for services appropriate to the use. The easement covers all works associated with the access in accordance with Planning scheme policy - Integrated design. | No example provided. |  |  |
| PO24 The layout of the development does not compromise: - the development of the road network in the area; - the function or safety of the road network; - the capacity of the road network. Note - The road hierarchy is mapped on Overlay map - Road hierarchy.. | E24.1 Direct vehicle access for residential development does not occur from arterial or sub-arterial roads or a motorway. Editor's note - Residential developments should consider amalgamation with the lot to the rear and gaining access via a laneway.. Note - The road hierarchy is mapped on Overlay map - Road hierarchy.. |  |  |
|  | E24.2 The development provides for the extension of the road network in the area in accordance with Council’s road network planning. |  |  |
|  | E24.3 The development does not compromise future road widening of frontage roads in accordance with the relevant standard and Council’s road planning. |  |  |
|  | E24.4 The lot layout allows forward access to and from the site. |  |  |
| PO25 Safe access is provided for all vehicles required to access the site. | E25.1 Site access and driveways are designed and located in accordance with: - Where for a Council-controlled road, AS/NZS2890.1 section 3; or - Where for a State-Controlled road, the Safe Intersection Sight Distance requirements in AustRoads and the appropriate IPWEAQ standard drawings, or a copy of a Transport Infrastructure Act 1994, section 62 approval. |  |  |
|  | E25.2 Internal driveways and access ways are designed and constructed in accordance with AS/NZS2890.1 Parking Facilities – Off street car parking and the relevant standards in Planning scheme policy - Integrated design. Note - This includes queue lengths (refer to Schedule 8 Service vehicle requirements), pavement widths and construction.. |  |  |
|  | E25.3 Access driveways, manoeuvring areas and loading facilities provide for service vehicles listed in Schedule 8 Service vehicle requirements for the relevant use. The on-site manoeuvring is to be in accordance with Schedule 8 Service vehicle requirements. |  |  |
| PO26 Upgrade works (whether trunk or non-trunk) are provided where necessary to: - ensure the type or volume of traffic generated by the development does not have a negative impact on the external road network; - ensure the orderly and efficient continuation of the active transport network; - ensure the site frontage is constructed to a suitable urban standard generally in accordance with Planning scheme policy - Integrated design. Note - An Integrated Transport Assessment (ITA) may be required to demonstrate compliance with this performance outcome refer to Planning scheme policy - Integrated transport assessment for guidance on when an ITA is required. An ITA should be prepared in accordance with Planning scheme policy - Integrated transport assessment.. Note - The road network is mapped on Overlay map - Road hierarchy.. Note - The primary and secondary active transport network is mapped on Overlay map - Active transport.. Note - To demonstrate compliance with c. of this performance outcome, site frontage works where in existing road reserve (non-trunk) are to be designed and constructed as follows: - Where the street is partially established to an urban standard, match the alignment of existing kerb and channel and provide carriageway widening and underground drainage where required; or - Where the street is not established to an urban standard, prepare a design that demonstrates how the relevant features of the particular road as shown in the Planning scheme policy - Integrated Design can be achieved in the existing reserve.. Note - Refer to Planning scheme policy - Integrated design for road network and active transport network design standards.. | No example provided. |  |  |
| Stormwater |  |  |  |
| PO27 Stormwater run-off from the site is conveyed to a point of lawful discharge without causing nuisance or annoyance to any person, property or premises. Note - Refer to Planning scheme policy - Integrated design for details.. Note - A downstream drainage discharge report in accordance with Planning scheme policy - Stormwater management may be required to demonstrate achievement of this performance outcome.. Note - A watercourse as defined in the Water Act may be accepted as a lawful point of discharge providing the drainage discharge from the site does not increase the downstream flood levels during events up to and including the 1% AEP storm. An afflux of +20mm may be accepted on Council controlled land and road infrastructure. No worsening is ensured when stormwater is discharged into a catchment that includes State Transport Infrastructure.. | No example provided. |  |  |
| PO28 Stormwater generated from the development does not compromise the capacity of existing stormwater infrastructure downstream of the site. Note - A downstream drainage discharge report in accordance with Planning scheme policy - Stormwater management may be required to demonstrate achievement of this performance outcome.. | No example provided. |  |  |
| PO29 Stormwater quality management systems are designed and constructed to minimise the environmental impact of stormwater discharge on surface and underground receiving water quality and meet the design objectives in Tables A and B in Appendix 2 of the SPP. Note - A stormwater management plan prepared by a suitably qualified professional will be required in accordance with Planning scheme policy - Stormwater management.. | No example provided. |  |  |
| PO30 Easements for drainage purposes are provided over: - stormwater pipes located in freehold land if the pipe diameter exceeds 300mm; - overland flow paths where they cross more than one property boundary. Note - Refer to Planning scheme policy - Integrated design for details.. Note - Stormwater Drainage easement dimensions are provided in accordance with Section 3.8.5 of QUDM.. | No example provided. |  |  |
| Site works and construction management |  |  |  |
| PO31 The site and any existing structures are maintained in a tidy and safe condition. | No example provided. |  |  |
| PO32 All works on-site are managed to: - minimise as far as practicable, impacts on adjoining or adjacent premises and the streetscape in regard to erosion and sedimentation, dust, noise, safety and light; - minimise as far as possible, impacts on the natural environment; - ensure stormwater discharge is managed in a manner that does not cause nuisance or annoyance to any person or premises; - avoid adverse impacts on street trees and their critical root zone. | E32.1 Works incorporate temporary stormwater runoff, erosion and sediment controls and trash traps designed in accordance with the Urban Stormwater Quality Planning Guidelines, Planning scheme policy - Stormwater management and Planning scheme policy - Integrated design, including but not limited to the following: - stormwater is not discharged to adjacent properties in a manner that differs significantly from pre-existing conditions; - stormwater discharged to adjoining and downstream properties does not cause scour and erosion; - stormwater discharge rates do not exceed pre-existing conditions; - the 10% AEP storm event is the minimum design storm for all temporary diversion drains; and - the 50% AEP storm event is the minimum design storm for all silt barriers and sedimentation basins. |  |  |
|  | E32.2 Stormwater runoff, erosion and sediment controls are constructed prior to commencement of any clearing or earthworks and are maintained and adjusted as necessary at all times to ensure their ongoing effectiveness. Note - The measures are adjusted on-site to maximise their effectiveness.. |  |  |
|  | E32.3 The completed earthworks area is stabilised using turf, established grass seeding, mulch or sprayed stabilisation techniques to control erosion and sediment and dust from leaving the property. |  |  |
|  | E32.4 Where works are proposed in proximity to an existing street tree, an inspection and a root management plan is undertaken by a qualified arborist which demonstrates and ensures that no permanent damage is caused to the tree. |  |  |
| PO33 Dust suppression measures are implemented during soil disturbances and construction works to protect nearby premises from unreasonable dust impacts. | E33 No dust emissions extend beyond the boundaries of the site during soil disturbances and construction works. |  |  |
| PO34 All works on-site and the transportation of material to and from the site are managed to not negatively impact the existing road network, the amenity of the surrounding area or the streetscape. Note - Where the amount of imported or exported material is greater than 50m3, a haulage route must be identified and approved by Council.. | E34.1 Construction traffic including contractor car parking is controlled in accordance with a traffic management plan, prepared in accordance with the Manual of Uniform Traffic Control Devices (MUTCD) to ensure all traffic movements to and from the site are safe. |  |  |
|  | E34.2 All contractor car parking is either provided on the development site, or on an alternative site in the general locality which has been set aside for car parking. Contractors vehicles are generally not to be parked in existing roads. Note - A Traffic Management Plan may be required for the site in accordance with the Manual of Uniform Traffic Control Devices (MUTCD).. |  |  |
|  | E34.3 Any material dropped, deposited or spilled on the road(s) as a result of construction processes associated with the site are to be cleaned at all times. |  |  |
| PO35 All disturbed areas are rehabilitated at the completion of construction. Note - Refer to Planning scheme policy - Integrated design for details.. | E35 At completion of construction all disturbed areas of the site are to be: - topsoiled with a minimum compacted thickness of fifty (50) millimetres; - grassed. Note - These areas are to be maintained during any maintenance period to maximise grass coverage from grass seeding of these areas.. |  |  |
| PO36 The clearing of vegetation on-site: - is limited to the area of infrastructure works, building areas and other necessary areas for the works; and - includes the removal of declared weeds and other materials which are detrimental to the intended use of the land; - is disposed of in a manner which minimises nuisance and annoyance to existing premises. Note - No burning of cleared vegetation is permitted.. | E36.1 All native vegetation to be retained on-site is temporarily fenced or protected prior to and during development works. Note - No parking of vehicles of storage of machinery or goods is to occur in these areas during development works.. |  |  |
|  | E36.2 Disposal of materials is managed in one or more of the following ways: - all cleared vegetation, declared weeds, stumps, rubbish, car bodies, scrap metal and the like are removed and disposed of in a Council land fill facility; or - all native vegetation with a diameter below 400mm is to be chipped and stored on-site. Note - The chipped vegetation must be stored in an approved location, preferably a park or public land.. |  |  |
| PO37 Any alteration or relocation in connection with or arising from the development to any service, installation, plant, equipment or other item belonging to or under the control of the telecommunications authority, electricity authorities, the Council or other person engaged in the provision of public utility services is to be carried with the development and at no cost to Council. | No example provided. |  |  |
| Earthworks |  |  |  |
| PO38 On-site earthworks are designed to consider the visual and amenity impact as they relate to: - the natural topographical features of the site; - short and long-term slope stability; - soft or compressible foundation soils; - reactive soils; - low density or potentially collapsing soils; - existing fill and soil contamination that may exist on-site; - the stability and maintenance of steep rock slopes and batters; - excavation (cut) and fill and impacts on the amenity of adjoining lots (e.g. residential). Note - Filling or excavation works are to be completed within six months of the commencement date.. | E38.1 All cut and fill batters are provided with appropriate scour, erosion protection and run-off control measures including catch drains at the top of batters and lined batter drains as necessary. |  |  |
|  | E38.2 Stabilisation measures are provided, as necessary, to ensure long-term stability and low maintenance of steep rock slopes and batters. |  |  |
|  | E38.3 Inspection and certification of steep rock slopes and batters is required by a suitably qualified and experienced RPEQ. |  |  |
|  | E38.4 All fill batters steeper than 1 (V) in 6 (H) on residential lots are fully turfed to prevent scour and erosion. |  |  |
|  | E38.5 All filling or excavation is contained on-site. |  |  |
|  | E38.6 All fill placed on-site is: - limited to that required for the necessary approved use; - clean and uncontaminated (i.e. no building waste, concrete, green waste or contaminated material etc. is used as fill). |  |  |
|  | E38.7 The site is prepared and the fill placed on-site in accordance with AS3798. Note - The fill is to be inspected and tested in accordance with Planning scheme policy - Operational works inspection, maintenance and bonding procedures.. |  |  |
| PO39 Embankments are stepped, terraced and landscaped to not adversely impact on the visual amenity of the surrounding area. | E39 Any embankments more than 1.5 metres in height are stepped, terraced and landscaped. Figure - Embankment |  |  |
| PO40 Filling or excavation is undertaken in a manner that: - does not adversely impact on a Council or public sector entity maintained infrastructure or any drainage feature on, or adjacent to the land; - does not preclude reasonable access to a Council or public sector entity maintained infrastructure or any drainage feature on, or adjacent to the land for monitoring, maintenance or replacement purposes. Note - Public sector entity as defined in the Sustainable Planning Act 2009.. | E40.1 No filling or excavation is undertaken in an easement issued in favour of Council or a public sector entity. Note - Public sector entity as defined in the Sustainable Planning Act 2009.. |  |  |
|  | E40.2 Filling or excavation that would result in any of the following is not carried out on-site: - a reduction in cover over any Council or public sector entity infrastructure service to less than 600mm; - an increase in finished surface grade over, or within 1.5m on each side of, the Council or public sector entity infrastructure above that which existed prior to the earthworks being undertaken. Note - Public sector entity as defined in the Sustainable Planning Act 2009.. |  |  |
| PO41 Filling or excavation does not result in land instability. Note - Steep rock slopes and batters are inspected and certified for long-term stability by a suitably qualified and experienced geotechnical engineer with RPEQ qualifications. Stabilisation measures are provided, as necessary, to ensure long-term stability and low maintenance.. | No example provided. |  |  |
| PO42 Development does not result in - adverse impacts on the hydrological and hydraulic capacity of the waterway or floodway; - increased flood inundation outside the site; - any reduction in the flood storage capacity in the floodway; - and any clearing of native vegetation. Note - To demonstrate compliance with this outcome, Planning Scheme Policy - Stormwater Management provides guidance on the preparation of a site based stormwater management plan by a suitably qualified professional. Refer to Planning scheme policy - Integrated design for guidance on infrastructure design and modelling requirements.. | No example provided. |  |  |
| Retaining walls and structures |  |  |  |
| PO43 All earth retaining structures provide a positive interface with the streetscape and minimise impacts on the amenity of adjoining residents. | E43 Earth retaining structures: - are not constructed of boulder rocks or timber; - where height is no greater than 900mm, are provided in accordance with Figure - Retaining on a boundary; Figure - Retaining on boundary - where height is greater than 900mm but no greater than 1.5m, are to be setback at least the equivalent height of the retaining structure from any property boundary; - where height is greater than 1.5m, are to be setback and stepped 1.5m vertical: 1.5m horizontal, terraced, landscaped and drained as shown below. Figure - Cut Figure - Fill |  |  |
| Fire Services Note - The provisions under this heading only apply if: - the development is for, or incorporates: - reconfiguring a lot for a community title scheme creating 1 or more vacant lots; or - material change of use for 2 or more sole occupancy units on the same lot, or within the same community titles scheme; or - material change of use for a Tourist park(84) with accommodation in the form of caravans or tents; or - material change of use for outdoor sales(54), outdoor processing or outdoor storage where involving combustible materials. AND - none of the following exceptions apply: - the distributor-retailer for the area has indicated, in its netserv plan, that the premises will not be served by that entity’s reticulated water supply; or - every part of the development site is within 60m walking distance of an existing fire hydrant on the distributor-retailer’s reticulated water supply network, measured around all obstructions, either on or adjacent to the site.. Note - The provisions under this heading do not apply to buildings that are required by the Building Code of Australia to have a fire hydrant system complying with Australian Standard AS 2419.1 (2005) – Fire Hydrant Installations or other fire fighting facilities which provide equivalent protection.. |
| PO44 Development incorporates a fire fighting system that: - satisfies the reasonable needs of the fire fighting entity for the area; - is appropriate for the size, shape and topography of the development and its surrounds; - is compatible with the operational equipment available to the fire fighting entity for the area; - considers the fire hazard inherent in the materials comprising the development and their proximity to one another; - considers the fire hazard inherent in the surrounds to the development site; - is maintained in effective operating order. Note - The Queensland Fire and Emergency Services is the entity currently providing the fire fighting function for the urban areas of the Moreton Bay Region.. | E44.1 External fire hydrant facilities are provided on site to the standard prescribed under the relevant parts of Australian Standard AS 2419.1 (2005) – Fire Hydrant Installations. Note - For this requirement for accepted development the following are the relevant parts of AS 2419.1 (2005) that may be applicable: - in regard to the form of any fire hydrant - Part 8.5 and Part 3.2.2.1, with the exception that for Tourist parks(84) or development comprised solely of dwellings and their associated outbuildings, single outlet above-ground hydrants or suitably signposted in-ground hydrants would be an acceptable alternative; - in regard to the general locational requirements for fire hydrants - Part 3.2.2.2 (a), (e), (f), (g) and (h) as well as Appendix B of AS 2419.1 (2005); - in regard to the proximity of hydrants to buildings and other facilities - Part 3.2.2.2 (b), (c) and (d), with the exception that: - for dwellings and their associated outbuildings, hydrant coverage need only extend to the roof and external walls of those buildings; - for caravans and tents, hydrant coverage need only extend to the roof of those tents and caravans; - for outdoor sales(54), processing or storage facilities, hydrant coverage is required across the entire area of the outdoor sales(54), outdoor processing and outdoor storage facilities; - in regard to fire hydrant accessibility and clearance requirements - Part 3.5 and, where applicable, Part 3.6.. |  |  |
|  | E44.2 A continuous path of travel having the following characteristics is provided between the vehicle access point to the site and each external fire hydrant and hydrant booster point on the land: - an unobstructed width of no less than 3.5m; - an unobstructed height of no less than 4.8m; - constructed to be readily traversed by a 17 tonne HRV fire brigade pumping appliance; - an area for a fire brigade pumping appliance to stand within 20m of each fire hydrant and 8m of each hydrant booster point. |  |  |
|  | E44.3 On-site fire hydrant facilities are maintained in effective operating order in a manner prescribed in Australian Standard AS1851 (2012) – Routine service of fire protection systems and equipment. |  |  |
| PO45 On-site fire hydrants that are external to buildings, as well as the available fire fighting appliance access routes to those hydrants, can be readily identified at all times from, or at, the vehicular entry point to the development site. | E45 For development that contains on-site fire hydrants external to buildings: - those external hydrants can be seen from the vehicular entry point to the site; or - a sign identifying the following is provided at the vehicular entry point to the site: - the overall layout of the development (to scale); - internal road names (where used); - all communal facilities (where provided); - the reception area and on-site manager’s office (where provided); - external hydrants and hydrant booster points; - physical constraints within the internal roadway system which would restrict access by fire fighting appliances to external hydrants and hydrant booster points. Note - The sign prescribed above, and the graphics used are to be: - in a form; - of a size; - illuminated to a level; which allows the information on the sign to be readily understood, at all times, by a person in a fire fighting appliance up to 4.5m from the sign.. |  |  |
| PO46 Each on-site fire hydrant that is external to a building is signposted in a way that enables it to be readily identified at all times by the occupants of any firefighting appliance traversing the development site. | E46 For development that contains on-site fire hydrants external to buildings, those hydrants are identified by way of marker posts and raised reflective pavement markers in the manner prescribed in the technical note Fire hydrant indication system produced by the Queensland Department of Transport and Main Roads. Note - Technical note Fire hydrant indication system is available on the website of the Queensland Department of Transport and Main Roads.. |  |  |

| Use specific criteria |  |  |  |
| --- | --- | --- | --- |
| Home based business (35) |  |  |  |
| PO47 The scale and intensity of the Home based business(35): - is compatible with the physical characteristics of the site and the character of the local area; - is able to accommodate anticipated car parking demand without negatively impacting the streetscape or road safety; - does not adversely impact on the amenity of the adjoining and nearby premises; - remains ancillary to the residential use of the dwelling; - does not create conditions which cause hazards or nuisances to neighbours or other persons not associated with the activity; - ensures employees and visitors to the site do not negatively impact the expected amenity of adjoining properties; - ensures service and delivery vehicles do not negatively impact the amenity of the area. | No example provided. |  |  |
| Major electricity infrastructure(43), Substation(80) and Utility installation(86) |  |  |  |
| PO48 The development does not have an adverse impact on the visual amenity of a locality and is: - high quality design and construction; - visually integrated with the surrounding area; - not visually dominant or intrusive; - located behind the main building line; - below the level of the predominant tree canopy or the level of the surrounding buildings and structures; - camouflaged through the use of colours and materials which blend into the landscape; - treated to eliminate glare and reflectivity; - landscaped; - otherwise consistent with the amenity and character of the zone and surrounding area. | E48.1 Development is designed to minimise surrounding land use conflicts by ensuring infrastructure, buildings, structures and other equipment: - are enclosed within buildings or structures; - are located behind the main building line; - have a similar height, bulk and scale to the surrounding fabric; - have horizontal and vertical articulation applied to all exterior walls. |  |  |
|  | E48.2 A minimum 3m wide strip of dense planting is provided around the outside of the fenced area, between the development and street frontage, side and rear boundaries. |  |  |
| PO49 Infrastructure does not have an impact on pedestrian health and safety. | E49 Access control arrangements: - do not create dead-ends or dark alleyways adjacent to the infrastructure; - minimise the number and width of crossovers and entry points; - provide safe vehicular access to the site; - do not utilise barbed wire or razor wire. |  |  |
| PO50 All activities associated with the development occur within an environment incorporating sufficient controls to ensure the facility: - generates no audible sound at the site boundaries where in a residential setting; or - meet the objectives as set out in the Environmental Protection (Noise) Policy 2008. | E50 All equipment which produces audible or non-audible sound is housed within a fully enclosed building incorporating sound control measures sufficient to ensure noise emissions meet the objectives as set out in the Environmental Protection (Noise) Policy 2008. |  |  |
| Sales office (72) |  |  |  |
| PO51 The Sales office(72) is designed to: - provide functional and safe access, manoeuvring areas and car parking spaces for the number and type of vehicles anticipated to access the site; - complement the streetscape character while maintaining surveillance between buildings and public spaces; - be temporary in nature. Note - Refer to Planning scheme policy - Residential design for access and crossover requirements.. | No example provided. |  |  |
| Telecommunications facility (81) Editor's note - In accordance with the Federal legislation Telecommunications facilities (81)must be constructed and operated in a manner that will not cause human exposure to electromagnetic radiation beyond the limits outlined in the Radiocommunications (Electromagnetic Radiation - Human Exposure) Standard 2003 and Radio Protection Standard for Maximum Exposure Levels to Radiofrequency Fields - 3Khz to 300Ghz.. |
| PO52 Telecommunications facilities(81) are co-located with existing telecommunications facilities(81), Utility installation(86), Major electricity infrastructure(43) or Substation(80) if there is already a facility in the same coverage area. | E52.1 New telecommunication facilities(81) are co-located on existing towers with new equipment shelter and associated structures positioned adjacent to the existing shelters and structures. |  |  |
|  | E52.2 If not co-located with an existing facility, all co-location opportunities have been investigated and fully exhausted within a 2km radius of the site. |  |  |
| PO53 A new Telecommunications facility(81) is designed and constructed to ensure co-masting or co-siting with other carriers both on the tower or pole and at ground level is possible in the future. | E53 A minimum of 45m2 is available at ground level to allow for additional equipment shelters and associated structures for the purpose of co-locating on the proposed facility. |  |  |
| PO54 Telecommunications facilities(81) do not conflict with lawful existing land uses both on and adjoining the site. | E54 The development results in no net reduction in the minimum quantity and standard of landscaping, private or communal open space or car parking spaces required under the planning scheme or under an existing development approval. |  |  |
| PO55 The Telecommunications facility(81) does not have an adverse impact on the visual amenity of a locality and is: - high quality design and construction; - visually integrated with the surrounding area; - not visually dominant or intrusive; - located behind the main building line; - below the level of the predominant tree canopy or the level of the surrounding buildings and structures; - camouflaged through the use of colours and materials which blend into the landscape; - treated to eliminate glare and reflectivity; - landscaped; - otherwise consistent with the amenity and character of the zone and surrounding area. | E55.1 Where in an urban area, the development does not protrude more than 5m above the level of the existing treeline, prominent ridgeline or building rooftops in the surrounding townscape. |  |  |
|  | E55.2 In all other areas towers do not exceed 35m in height. |  |  |
|  | E55.3 Towers, equipment shelters and associated structures are of a design, colour and material to: - reduce recognition in the landscape; - reduce glare and reflectivity. |  |  |
|  | E55.4 All structures and buildings are setback behind the main building line and a minimum of 10m from side and rear boundaries, except where in the Industry and Extractive industry zones, the minimum side and rear setback is 3m. Where there is no established building line the facility is located at the rear of the site. |  |  |
|  | E55.5 The facility is enclosed by security fencing or by other means to ensure public access is prohibited. |  |  |
|  | E55.6 A minimum 3m wide strip of dense planting is provided around the perimeter of the fenced area, between the facility and street frontage and adjoining uses. Note - Landscaping is provided in accordance with Planning scheme policy - Integrated design.. Note - Council may require a detailed landscaping plan, prepared by a suitably qualified person, to ensure compliance with Planning scheme policy - Integrated design.. |  |  |
| PO56 Lawful access is maintained to the site at all times that does not alter the amenity of the landscape or surrounding uses. | E56 An Access and Landscape Plan demonstrates how 24 hour vehicular access will be obtained and maintained to the facility in a manner that is appropriate to the site’s context. |  |  |
| PO57 All activities associated with the development occur within an environment incorporating sufficient controls to ensure the facility generates no audible sound at the site boundaries where in a residential setting. | E57 All equipment comprising the Telecommunications facility(81) which produces audible or non-audible sound is housed within a fully enclosed building incorporating sound control measures sufficient to ensure no noise from this equipment can be heard, or felt at the site boundary. |  |  |
| Retail, commercial and community uses |  |  |  |
| PO58 Community activities: - are located to: - cluster with other non-residential activities to form a neighbourhood hub (this may include being located within or adjacent to an existing neighbourhood hub); or - if establishing a new neighbourhood hub (as described in the PO below); be on a main street; - are located on allotments that have appropriate area and dimensions for the sitting of: - buildings and structures; - vehicle servicing, deliveries, parking, manoeuvring and circulation; - landscaping and open space including buffering; - are of a small scale, having regard to the surrounding character; - are serviced by public transport; - do not negatively impact adjoining residents or the streetscape. | No example provided. |  |  |
| PO59 Retail and commercial activities do not establish in this precinct unless: - forming part of a existing or new neighbourhood hub on a site identified on Overlay map - Community activities and neighbourhood hubs; or - forming a new neighbourhood hub where the urban neighbourhood precinct does not adjoin a higher order or district centre (e.g. Clontarf, Woody Point, Scarborough) or where adjoining or opposite a train station; or - forming part of a mixed-use building with residential uses; or - for a corner store. | No example provided. |  |  |
| PO60 Retail and commercial uses may establish as standalone uses (not part of a neighbourhood hub) where: - for a corner store or forming part of a mixed use building; - having a maximum GFA of 250m2; - the building adjoins the street frontage and has its main pedestrian entrance from the street frontage. | No example provided. |  |  |
| PO61 Retail and commercial uses within a neighbourhood hub are of a scale that provide for the convenience needs or localised services of the immediate neighbourhood and do not constitute the scale or function of a Local centre. Note - For the function and scale of a Local centre refer to Table 6.2.1.1 Moreton Bay centres network.. | E61 Retail and commercial uses within a neighbourhood hub consist of no more than: - 1 small format supermarket with a maximum GFA of 1200m2; - 10 small format retail or commercial tenancies with a maximum GFA of 100m2 each. |  |  |
| PO62 Non-residential uses address and activate streets and public spaces by: - ensuring buildings and individual tenancies address street frontage(s), civic space and other areas of pedestrian movement; - new buildings adjoin or are within 3m of the primary street frontage(s), civic space or public open space; - locating car parking areas behind or under buildings to not dominate the street environment; - establishing and maintaining interaction, pedestrian activity and casual surveillance through appropriate land uses and building design (e.g. The use of windows or glazing and avoiding blank walls with the use of sleeving); - providing visual interest to the façade (e.g. Windows or glazing, variation in colour, materials, finishes, articulation, recesses or projections); - establishing and maintaining human scale. | No example provided. |  |  |
| PO63 All buildings exhibit a high standard of design and construction, which: - add visual interest to the streetscape (e.g. variation in materials, patterns, textures and colours, cantilevered awning); - enable differentiation between buildings; - contribute to a safe environment; - incorporate architectural features within the building facade at the street level to create human scale (e.g. cantilevered awning); - include building entrances that are readily identifiable from the road frontage; - locate and orientate to favour active and public transport usage by connecting to pedestrian footpaths on the street frontage and adjoining sites; - incorporate appropriate acoustic treatments, having regard to any adjoining residential uses; - facilitate casual surveillance of all public spaces. | No example provided. |  |  |
| PO64 Development provides functional and integrated car parking and vehicle access, that: - prioritises the movement and safety of pedestrians between the street frontage and the entrance to the building; - provides safety and security of people and property at all times; - does not impede active transport options; - does not impact on the safe and efficient movement of traffic external to the site; - is consolidated and shared with adjoining sites wherever possible. | No example provided. |  |  |
| PO65 The safety and efficiency of pedestrian movement is prioritised in the design of car parking areas through providing pedestrian paths in car parking areas that are: - located along the most direct route between building entrances, car parks and adjoining uses; - protected from vehicle intrusion through the use of physical and visual separation (e.g. wheel stops, trees etc); - are of a width to allow safe and efficient access for prams and wheelchairs. | No example provided. |  |  |
| PO66 The number of car parking spaces is managed to: - avoid significant impacts on the safety and efficiency of the road network; - avoid an oversupply of car parking spaces; - avoid the visual impact of large areas of open car parking from road frontages and public areas; - promote active and public transport options; - promote innovative solutions, including on-street parking and shared parking areas. Note - Refer to Planning scheme policy - Integrated transport assessment for guidance on how to achieve compliance with this outcome.. | E66.1 Car parking is provided in accordance with Table 6.2.6.4.5 ‘Car parking spaces’. Note - The above rates exclude car parking spaces for people with a disability required by Disability Discrimination Act 1992 or the relevant disability discrimination legislation and standards.. |  |  |
|  | E66.2 All car parking areas are designed and constructed in accordance with Australian Standard AS2890.1. |  |  |
| PO67 - End of trip facilities are provided for employees or occupants, in the building or on-site within a reasonable walking distance, and include: - adequate bicycle parking and storage facilities; and - adequate provision for securing belongings; and - change rooms that include adequate showers, sanitary compartments, wash basins and mirrors. - Notwithstanding a. there is no requirement to provide end of trip facilities if it would be unreasonable to provide these facilities having regard to: - the projected population growth and forward planning for road upgrading and development of cycle paths; or - whether it would be practical to commute to and from the building on a bicycle, having regard to the likely commute distances and nature of the terrain; or - the condition of the road and the nature and amount of traffic potentially affecting the safety of commuters. Editor's note - The intent of b above is to ensure the requirements for bicycle parking and end of trip facilities are not applied in unreasonable circumstances. For example these requirements should not, and do not apply in the Rural zone or the Rural residential zone etc.. Editor's note - This performance outcome is the same as the Performance Requirement prescribed for end of trip facilities under the Queensland Development Code. For development incorporating building work, that Queensland Development Code performance requirement cannot be altered by a local planning instrument and has been reproduced here solely for information purposes. Council’s assessment in its building work concurrence agency role for end of trip facilities will be against the performance requirement in the Queensland Development Code. As it is subject to change at any time, applicants for development incorporating building work should ensure that proposals that do not comply with the examples under this heading meet the current performance requirement prescribed in the Queensland Development Code.. | E67.1 Minimum bicycle parking facilities are provided in accordance with the table below (rounded up to the nearest whole number). Use; Minimum Bicycle Parking. Residential uses comprised of dwellings; Minimum 1 space per dwelling. All other residential uses; Minimum 1 space per 2 car parking spaces identified in Schedule 7 – car parking. Non-residential uses; Minimum 1 space per 200m2 of GFA. Editor's note - The examples for end of trip facilities prescribed under the Queensland Development Code permit a local planning instrument to prescribe facility levels higher than the default levels identified in those acceptable solutions. This example is a combination of the default levels set for end of trip facilities in the Queensland Development Code and the additional facilities required by Council.. |  |  |
|  | E67.2 Bicycle parking is: - provided in accordance with Austroads (2008), Guide to Traffic Management - Part 11: Parking; - protected from the weather by its location or a dedicated roof structure; - located within the building or in a dedicated, secure structure for residents and staff; - adjacent to building entrances or in public areas for customers and visitors. Note - Bicycle parking structures are to be constructed to the standards prescribed in AS2890.3.. Note - Bicycle parking and end of trip facilities provided for residential and non-residential activities may be pooled, provided they are within 100 metres of the entrance to the building.. Editor's note - The examples for end of trip facilities prescribed under the Queensland Development Code permit a local planning instrument to prescribe facility levels higher than the default levels identified in those acceptable solutions. This example is an amalgamation of the default levels set for end of trip facilities in the Queensland Development Code and the additional facilities required by Council.. |  |  |
|  | E67.3 For non-residential uses, storage lockers: - are provide at a rate of 1.6 per bicycle parking space (rounded up to the nearest whole number); - have minimum dimensions of 900mm (height) x 300mm (width) x 450mm (depth). Note - Storage lockers may be pooled across multiple sites and activities when within 100 metres of the entrance to the building and within 50 metres of bicycle parking and storage facilities.. Editor's note - The examples for end of trip facilities prescribed under the Queensland Development Code permit a local planning instrument to prescribe facility levels higher than the default levels identified in those acceptable solutions. This example is an amalgamation of the default levels set for end of trip facilities in the Queensland Development Code and the additional facilities required by Council.. |  |  |
|  | E67.4 For non-residential uses, changing rooms: - are provided at a rate of 1 per 10 bicycle parking spaces; - are fitted with a lockable door or otherwise screened from public view; - are provided with shower(s), sanitary compartment(s) and wash basin(s) in accordance with the table below: Bicycle spaces provided; Male/ Female; Change rooms required; Showers required; Sanitary compartments required; Washbasins required. 1-5; Male and female; 1 unisex change room; 1; 1 closet pan; 1. 6-19; Female; 1; 1; 1 closet pan; 1. 20 or more; Male; 1; 1; 1 closet pan; 1. Female; 1; 2, plus 1 for every 20 bicycle spaces provided thereafter; 2 closet pans, plus 1 sanitary compartment for every 60 bicycle parking spaces provided thereafter; 1, plus 1 for every 60 bicycle parking spaces provided thereafter. Male; 1; 2, plus 1 for every 20 bicycle spaces provided thereafter; 1 urinal and 1 closet pans, plus 1 sanitary compartment at the rate of 1 closet pan or 1 urinal for every 60 bicycle space provided thereafter; 1, plus 1 for every 60 bicycle parking spaces provided thereafter. Note - All showers have a minimum 3-star Water Efficiency Labelling and Standards (WELS) rating shower head. Note - All sanitary compartments are constructed in compliance with F2.3 (e) and F2.5 of BCA (Volume 1).. - are provided with: - a mirror located above each wash basin; - a hook and bench seating within each shower compartment; - a socket-outlet located adjacent to each wash basin. Note - Change rooms may be pooled across multiple sites, residential and non-residential activities when within 100 metres of the entrance to the building and within 50 metres of bicycle parking and storage facilities. Editor's note - The examples for end of trip facilities prescribed under the Queensland Development Code permit a local planning instrument to prescribe facility levels higher than the default levels identified in those acceptable solutions. This example is an amalgamation of the default levels set for end of trip facilities in the Queensland Development Code and the additional facilities required by Council.. |  |  |
| PO68 Loading and servicing areas: - are not visible from the street frontage; - are integrated into the design of the building; - include screening and buffers to reduce negative impacts on adjoining sensitive land uses; - where possible loading and servicing areas are consolidated and shared with adjoining sites. | No example provided. |  |  |
| PO69 Bins and bin storage areas are provided, designed and managed in accordance with Planning scheme policy – Waste. | No example provided. |  |  |
| PO70 On-site landscaping is provided, that: - is incorporated into the design of the development; - reduces the dominance of car parking and servicing areas from the street frontage; - retains mature trees wherever possible; - does not create safety or security issues by creating potential concealment areas or interfering with sight lines; - maintains the achievement of active frontages and sight lines for casual surveillance. Note - All landscaping is to accord with Planning scheme policy - Integrated design.. | No example provided. |  |  |
| PO71 Surveillance and overlooking are maintained between the road frontage and the main building line. | E71 No fencing is provided forward of the building line. |  |  |
| PO72 Lighting is designed to provide adequate levels of illumination to public and communal spaces to maximise safety and minimise adverse impacts on residential and other sensitive land uses. | No example provided. |  |  |
| PO73 The hours of operation minimise adverse amenity impacts on adjoining sensitive land uses. | E73 Hours of operation do not exceed 6:00am to 9:00pm Monday to Sunday. |  |  |

| Values and constraints criteria Note - The relevant values and constraints criteria do not apply where the development is consistent with a current Development permit for Reconfiguring a lot or Material change of use or Operational work, where that approval has considered and addressed (e.g. through a development footprint plan (or similar in the case of Landslide hazard) or conditions of approval) the identified value or constraint under this planning scheme.. |
| --- |
| Acid sulfate soils - (refer Overlay map - Acid sulfate soils to determine if the following assessment criteria apply) Note - To demonstrate achievement of the performance outcome, an Acid sulfate soils (ASS) investigation report and soil management plan is prepared by a qualified engineer. Guidance for the preparation an ASS investigation report and soil management plan is provided in Planning scheme policy - Acid sulfate soils.. |
| PO74 Development avoids disturbing acid sulfate soils. Where development disturbs acid sulfate soils, development: - is managed to avoid or minimise the release of surface or groundwater flows containing acid and metal contaminants into the environment; - protects the environmental and ecological values and health of receiving waters; - protects buildings and infrastructure from the effects of acid sulfate soils. | E74 Development does not involve: - excavation or otherwise removing of more than 100m3 of soil or sediment where below than 5m Australian Height datum AHD; or - filling of land of more than 500m3 of material with an average depth of 0.5m or greater where below the 5m Australian Height datum AHD. |  |  |
| Environmental areas (refer Overlay map - Environmental areas to determine if the following assessment criteria apply) Note – The following are excluded from the native vegetation clearing provisions of this planning scheme: - Clearing of native vegetation located within an approved development footprint; - Clearing of native vegetation within 10m from a lawfully established building reasonably necessary for emergency access or immediately required in response to an accident or emergency; - Clearing of native vegetation reasonably necessary to remove or reduce the risk vegetation poses to serious personal injury or damage to infrastructure; - Clearing of native vegetation reasonably necessary to construct and maintain a property boundary fence and not exceed 4m in width either side of the fence where in the Rural, Rural residential and Environmental Management and Conservation zones. In any other zone, clearing is not to exceed 2m in width either side of the fence; - Clearing of native vegetation reasonably necessary for the purpose of maintenance or works within a registered easement for public infrastructure or drainage purposes; - Clearing of native vegetation in accordance with a bushfire management plan prepared by a suitably qualified person, submitted to and accepted by Council; - Clearing of native vegetation associated with removal of recognised weed species, maintaining existing open pastures and cropping land, windbreaks, lawns or created gardens; - Grazing of native pasture by stock; - Native forest practice where accepted development under Part 1, 1.7.7 Accepted development. Note - Definition for native vegetation is located in Schedule 1 Definitions. Note - Native vegetation subject to this criteria primarily comprises of matters of national environmental significance (MNES), matters of state environmental significance (MSES). They also comprise some matters of local environmental significance (MLES). A MLES is defined in Schedule 1.2, Administrative definitions. A list of the elements that apply to the mapped MSES and MLES is provided in Appendix 1 of the Planning scheme policy - Environmental areas. Editors' Note - The accuracy of overlay mapping can be challenged through the development application process (code assessable development) or by way of a planning scheme amendment. See Council's website for details.. Note - To demonstrate achievement of the performance outcome, an ecological assessment, vegetation management plan and fauna management plan, as required, are prepared by a suitably qualified person. Guidance for the preparation of above mentioned reports is provided in Planning scheme policy - Environmental areas.. |
| Vegetation clearing, ecological value and connectivity |  |  |  |
| PO75 Development avoids locating in a High Value Area or a Value Offset Area. Where it is not practicable or reasonable for development to avoid establishing in these areas, development must ensure that: - the quality and integrity of the biodiversity and ecological values inherent to a High Value Area and a Value Offset Area is maintained and not lost or degraded; - on-site mitigation measures, mechanisms or processes are in place demonstrating the quality and integrity of the biodiversity and ecological values inherent to a High Value Area and a Value Offset Area are maintained. For example, this can be achieved through replacement, restoration or rehabilitation planting as part of any proposed covenant, the development of a Vegetation Management Plan, a Fauna Management Plan, and any other on-site mitigation options identified in the Planning scheme policy - Environmental areas*. * Editor's note - This is not a requirement for an environmental offset under the Environmental Offsets Act 2014.. | No example provided. |  |  |
| PO76 Development provides for safe, unimpeded, convenient and ongoing wildlife movement and establishes and maintains habitat connectivity by: - retaining habitat trees; - providing contiguous patches of habitat; - provide replacement and rehabilitation planting to improve connectivity; - avoiding the creation of fragmented and isolated patches of habitat; - providing wildlife movement infrastructure. Editor's note - Wildlife movement infrastructure may include refuge poles, tree boulevarding, ‘stepping stone’ vegetation plantings, tunnels, appropriate wildlife fencing; culverts with ledges, underpasses, overpasses, land bridges and rope bridges. Further information is provided in Planning scheme policy – Environmental areas.. | No example provided. |  |  |
| Vegetation clearing and habitat protection |  |  |  |
| PO77 Development ensures that the biodiversity quality and integrity of habitats is not adversely impacted upon but maintained and protected. | No example provided. |  |  |
| PO78 Development does not result in the net loss or degradation of habitat value in a High Value Area or a Value Offset Area. Where development does result in the loss or degradation of habitat value, development will: - rehabilitate, revegetate, restore and enhance an area to ensure it continues to function as a viable and healthy habitat area; - provide replacement fauna nesting boxes in the event of habitat tree loss in accordance with Planning scheme policy - Environmental areas; - undertake rehabilitation, revegetation and restoration in accordance with the South East Queensland Ecological Restoration Framework. | No example provided. |  |  |
| PO79 Development ensures safe, unimpeded, convenient and ongoing wildlife movement and habitat connectivity by: - providing contiguous patches of habitat; - avoiding the creation of fragmented and isolated patches of habitat; - providing wildlife movement infrastructure; - providing replacement and rehabilitation planting to improve connectivity. | No example provided. |  |  |
| Vegetation clearing and soil resource stability |  |  |  |
| PO80 Development does not: - result in soil erosion or land degradation; - leave cleared land exposed for an unreasonable period of time but is rehabilitated in a timely manner. | No example provided. |  |  |
| Vegetation clearing and water quality |  |  |  |
| PO81 Development maintains or improves the quality of groundwater and surface water within, and downstream, of a site by: - ensuring an effective vegetated buffers and setbacks from waterbodies is retained to achieve natural filtration and reduce sediment loads; - avoiding or minimising changes to landforms to maintain hydrological water flows; - adopting suitable measures to exclude livestock from entering a waterbody where a site is being used for animal husbandry(4) and animal keeping(5) activities. | No example provided. |  |  |
| PO82 Development minimises adverse impacts of stormwater run-off on water quality by: - minimising flow velocity to reduce erosion; - minimising hard surface areas; - maximising the use of permeable surfaces; - incorporating sediment retention devices; - minimising channelled flow. | No example provided. |  |  |
| Vegetation clearing and access, edge effects and urban heat island effects |  |  |  |
| PO83 Development retains safe and convenient public access in a manner that does not result in the adverse edge effects or the loss or degradation of biodiversity values within the environment. | No example provided. |  |  |
| PO84 Development minimises potential adverse ‘edge effects’ on ecological values by: - providing dense planting buffers of native vegetation between a development and environmental areas; - retaining patches of native vegetation of greatest possible size where located between a development and environmental areas ; - restoring, rehabilitating and increasing the size of existing patches of native vegetation; - ensuring that buildings and access (public and vehicle) are setback as far as possible from environmental areas and corridors; - landscaping with native plants of local origin. Editor's note - Edge effects are factors of development that go to detrimentally affecting the composition and density of natural populations at the fringe of natural areas. Factors include weed invasion, pets, public and vehicle access, nutrient loads, noise and light pollution, increased fire frequency and changes in the groundwater and surface water flow.. | No example provided. |  |  |
| PO85 Development avoids adverse microclimate change and does not result in increased urban heat island effects. Adverse urban heat island effects are minimised by: - pervious surfaces; - providing deeply planted vegetation buffers and green linkage opportunities; - landscaping with local native plant species to achieve well-shaded urban places; - increasing the service extent of the urban forest canopy. | No example provided. |  |  |
| Vegetation clearing and Matters of Local Environmental Significance (MLES) environmental offsets |  |  |  |
| PO86 Where development results in the unavoidable loss of native vegetation within a Value Offset Area MLES waterway buffer or a Value Offset Area MLES wetland buffer, an environmental offset is required in accordance with the environmental offset requirements identified in Planning scheme policy - Environmental areas. Editor's note - For MSES Koala Offsets, the environmental offset provisions in schedule 11 of the Regulation, in combination with the requirements of the Environmental Offset Act 2014, apply.. | No example provided. |  |  |
| Extractive resources transport route (refer Overlay map - Extractive resources (transport route and buffer) to determine if the following assessment criteria apply) |  |  |  |
| PO87 Development: - does not increase in the number of people living in close proximity to a transport route and being subject to the adverse effects from the transportation route; - does not result in the establishment of uses that are incompatible with the operation of Extractive resources transport routes; - adopts design and location measures to satisfactorily mitigate the potential adverse impacts associated with transportation routes on sensitive land uses. Such measures include, but are not limited to: - locating the furthest distance possible from the transportation route; - habitable rooms being located the furthest from the transportation route; - shielding and screening private outdoor recreation space from the transportation routes. | E87 The following uses are not located within the 100m wide transport route buffer: - Caretaker’s accommodation(10), except where located in the Extractive industry zone; - Community residence(16); - Dual occupancy(21); - Dwelling house(22); - Dwelling unit(23); - Hospital(36); - Rooming accommodation(69); - Multiple dwelling(49); - Non-resident workforce accommodation(52); - Relocatable home park(62); - Residential care facility(65); - Resort complex(66); - Retirement facility(67); - Rural workers’ accommodation(71); - Short-term accommodation(77); - Tourist park(84). |  |  |
| PO88 Development: - does not adversely impact upon the efficient and effective transportation of extractive material along a transportation route; - ensures vehicle access and egress along transportation routes are designed and located to achieve a high degree of safety, having good visibility; - utilises existing vehicle access points and where existing vehicle access points are sub-standard or poorly formed, they are upgraded to an appropriate standard. | E88.1 Development does not create a new vehicle access point onto an Extractive resources transport route. |  |  |
|  | E88.2 A vehicle access point is located, designed and constructed in accordance with Planning scheme policy - Integrated design. |  |  |
| Heritage and landscape character (refer Overlay map - Heritage and landscape character to determine if the following assessment criteria apply) Note - To assist in demonstrating achievement of heritage performance outcomes, a Cultural heritage impact assessment report is prepared by a suitably qualified person verifying the proposed development is in accordance with The Australia ICOMOS Burra Charter. Note - To assist in demonstrating achievement of this performance outcome, a Tree assessment report is prepared by a qualified arborist in accordance with Planning scheme policy – Heritage and landscape character. The Tree assessment report will also detail the measures adopted in accordance with AS 4970-2009 Protection of trees on development sites. Note - Places, including sites, objects and buildings having local cultural heritage significance, are identified on Overlay map - Heritage and landscape character and listed in Schedule 1 of Planning scheme policy - Heritage and landscape character. Places also having cultural heritage significance at a State level and being entered in the Queensland Heritage Register, are also identified in Schedule 1 of Planning scheme policy - Heritage and landscape character.. |
| PO89 Development will: - not diminish or cause irreversible damage to the cultural heritage values present on the site, and associated with a heritage site, object or building; - protect the fabric and setting of the heritage site, object or building; - be consistent with the form, scale and style of the heritage site, object or building; - utilise similar materials to those existing, or where this is not reasonable or practicable, neutral materials and finishes; - incorporate complementary elements, detailing and ornamentation to those present on the heritage site, object or building; - retain public access where this is currently provided. | E89 Development is for the preservation, maintenance, repair and restoration of a site, object or building of cultural heritage value. Note - A cultural heritage conservation management plan for the preservation, maintenance, repair and restoration of a site, object or building of cultural heritage value is prepared in accordance with Planning scheme policy - Heritage and landscape character. The plan is sent to, and approved by Council prior to the commencement of any preservation, maintenance, repair and restoration works.. |  |  |
| PO90 Demolition and removal is only considered where: - a report prepared by a suitably qualified conservation architect or conservation engineer demonstrates that the building is structurally unsound and is not reasonably capable of economic repair; or - demolition is confined to the removal of outbuildings, extensions and alterations that are not part of the original structure; or - limited demolition is performed in the course of repairs, maintenance or restoration; or - demolition is performed following a catastrophic event which substantially destroys the building or object. | No example provided. |  |  |
| PO91 Where development is occurring on land adjoining a site of cultural heritage value, the development is to be sympathetic to and consistent with the cultural heritage values present on the site and not result in their values being eroded, degraded or unreasonably obscured from public view. | No example provided. |  |  |
| PO92 Development does not adversely impact upon the health and vitality of significant trees. Where development occurs in proximity to a significant tree, construction measures and techniques as detailed in AS 4970-2009 Protection of trees on development sites are adopted to ensure a significant tree's health, wellbeing and vitality. Significant trees are only removed where they are in a poor state of health or where they pose a health and safety risk to persons or property. A Tree Assessment report prepared by a suitably qualified arborist confirming a tree's state of health is required to demonstrate achievement of this performance outcome. | E92 Development does: - not result in the removal of a significant tree; - not occur within 20m of a protected tree; - involve pruning of a tree in accordance with Australian Standard AS 4373-2007 – Pruning of Amenity Trees. |  |  |
| Infrastructure buffers (refer Overlay map - Infrastructure buffers to determine if the following assessment criteria apply) |  |  |  |
| PO93 Development within a Water supply buffer captures solid or liquid waste from all land use, development and activities is designed, constructed and managed to prevent the release of contaminants to surface water or groundwater bodies. | E93.1 Run-off and sediment from roadways and impervious surfaces within a Water supply buffer are intercepted and treated on-site to remove oil, grease, chemicals, silt, trace metals and nutrients such as nitrogen and phosphorous. |  |  |
|  | E93.2 Incineration or burial of waste within a Water supply buffer is not undertaken onsite. |  |  |
|  | E93.3 Solid waste within a Water supply buffer is collected and stored in weather proof, sealed waste receptacles, located in roofed and bunded areas, for disposal by a licenced contractor. |  |  |
|  | E93.4 Holding tanks within a Water supply buffer are used for all liquid waste and provide for the separation of oils/solvents and solids prior to pump-out and collection by a licenced contractor. |  |  |
|  | E93.5 Management, handling and storage of hazardous chemicals (including fuelling of vehicles) within a Water supply buffer, is undertaken in secured, climate controlled, weather proof, level and bunded enclosures. |  |  |
| PO94 On-site sewerage systems within a Water supply buffer are designed and operated to ensure there is no worsening or adverse impacts to health risks, environmental risks and water quality. Editor's Note - For guidance refer to the SEQ water Development Guidelines: Development Guidelines for Water Quality Management in Drinking Water Catchments 2012.. | E94 Secondary treated wastewater treatment systems within a Water supply buffer include: - emergency storage capable of holding 3-6 hours peak flow of treated effluent in the event of emergencies or overload with provision for de-sludging; - back up pump installation and backup power; - MEDLI modelling to determine irrigation rates and sizing of irrigation areas; - vegetated land application areas are not located in overland flow paths or on areas that perform groundwater recharge or discharge functions; and - wastewater collection and storage systems have a capacity to accommodate full load at peak times and includes temporary facilities. |  |  |
| PO95 Development within a Bulk water supply infrastructure buffer is located, designed and constructed to: - protect the integrity of the water supply pipeline; - maintain adequate access for any required maintenance or upgrading work to the water supply pipeline; | E95 Development: - does not involve the construction of any buildings or structures within a Bulk water supply infrastructure buffer; - involving a major hazard facility or environmentally relevant activity (ERA) is setback 30m from a Bulk water supply infrastructure buffer. |  |  |
| PO96 Development is located and designed to maintain required access to Bulk water supply infrastructure. | E96 Development does not restrict access to Bulk water supply infrastructure of any type or size, having regard to (among other things): - buildings or structures; - gates and fences; - storage of equipment or materials; - landscaping or earthworks or stormwater or other infrastructure. |  |  |
| PO97 Habitable rooms within an Electricity supply substation buffer are located a sufficient distance from substations(80) to avoid any potential adverse impacts on personal health and wellbeing from electromagnetic fields. Note - Habitable room is defined in the Building Code of Australia (Volume 1). | E97 Habitable rooms: - are not located within an Electricity supply substation buffer; and - proposed on a site subject to an Electricity supply substation(80)are acoustically insulted to achieve the noise levels listed in Schedule 1, Acoustic Quality Objectives, Environmental Protection (Noise) Policy 2008. Note - Habitable room is defined in the Building Code of Australia (Volume 1). |  |  |
| PO98 Habitable rooms within an Electricity supply substation buffer are acoustically insulated from the noise of a substation(80) to achieve the noise levels listed in Schedule 1 Acoustic Quality Objectives, Environmental Protection (Noise) Policy 2008 and provides a safe, healthy and disturbance free living environment. Note - To demonstrate achievement of the performance outcome, a noise impact assessment report is prepared by a suitably qualified person. Guidance to preparing an noise impact assessment report is provided in Planning scheme policy – Noise.. Note - Habitable room is defined in the Building Code of Australia (Volume 1). | No example provided. |  |  |
| PO99 Development within a Pumping station buffer is located, designed and constructed to: - ensure that odour or other air pollutant impacts on the amenity of the development met the air quality of objectives in the Environmental Protection (Air) Policy 2008; - ensure that noise impacts on the amenity of the development met the indoor noise objectives set out in the Environmental Protection (Noise) Policy 2008. | E99 Development does not involve the construction of any buildings or structures within a Pumping station buffer. |  |  |
| Overland flow path (refer Overlay map - Overland flow path to determine if the following assessment criteria apply) Note - The applicable river and creek flood planning levels associated with defined flood event (DFE) within the inundation area can be obtained by requesting a flood check property report from Council.. |
| PO100 Development: - minimises the risk to persons from overland flow; - does not increase the potential for damage from overland flow either on the premises or other premises, public land, watercourses, roads or infrastructure. | No example provided. |  |  |
| PO101 Development: - maintains the conveyance of overland flow predominantly unimpeded through the premises for any event up to and including the 1% AEP for the fully developed upstream catchment; - does not concentrate, intensify or divert overland flow onto an upstream, downstream or surrounding property. Note - A report from a suitably qualified Registered Professional Engineer Queensland is required certifying that the development does not increase the potential for significant adverse impacts on an upstream, downstream or surrounding premises.. Note - Reporting to be prepared in accordance with Planning scheme policy – Flood hazard, Coastal hazard and Overland flow.. | No example provided. |  |  |
| PO102 Development does not: - directly, indirectly or cumulatively cause any increase in overland flow velocity or level; - increase the potential for flood damage from overland flow either on the premises or other premises, public lands, watercourses, roads or infrastructure. Note - Open concrete drains greater than 1m in width are not an acceptable outcome, nor are any other design options that may increase scouring.. | No example provided. |  |  |
| PO103 Development ensures that public safety and the risk to the environment are not adversely affected by a detrimental impact of overland flow on a hazardous chemical located or stored on the premises. | E103 Development ensures that a hazardous chemical is not located or stored in an Overland flow path area. Note - Refer to the Work Health and Safety Act 2011 and associated Regulation and Guidelines, the Environmental Protection Act 1994 and the relevant building assessment provisions under the Building Act 1975 for requirements related to the manufacture and storage of hazardous substances.. |  |  |
| PO104 Development which is not in a Rural zone ensures that overland flow is not conveyed from a road or public open space onto a private lot. | E104 Development which is not in a Rural zone that an overland flow paths and drainage infrastructure is provided to convey overland flow from a road or public open space area away from a private lot. |  |  |
| PO105 Development ensures that inter-allotment drainage infrastructure, overland flow paths and open drains through private property cater for overland flows for a fully developed upstream catchment and are able to be easily maintained. Note - A report from a suitably qualified Registered Professional Engineer Queensland is required certifying that the development does not increase the potential for significant adverse impacts on an upstream, downstream or surrounding premises.. Note - Reporting to be prepared in accordance with Planning scheme policy – Flood hazard, Coastal hazard and Overland flow. | E105.1 Development ensures that roof and allotment drainage infrastructure is provided in accordance with the following relevant level as identified in QUDM: - Urban area – Level III; - Rural area – N/A; - Industrial area – Level V; - Commercial area – Level V. |  |  |
|  | E105.2 Development ensures that inter-allotment drainage infrastructure is designed to accommodate any event up to and including the 1% AEP for the fully developed upstream catchment. |  |  |
| PO106 Development protects the conveyance of overland flow such that an easement for drainage purposes is provided over: - a stormwater pipe if the nominal pipe diameter exceeds 300mm; - an overland flow path where it crosses more than one premises; - inter-allotment drainage infrastructure. Note - Refer to Planning scheme policy - Integrated design for details and examples.. Note - Stormwater Drainage easement dimensions are provided in accordance with Section 3.8.5 of QUDM.. | No example provided. |  |  |
| Additional criteria for development for a Park(57) |  |  |  |
| PO107 Development for a Park(57) ensures that the design and layout responds to the nature of the overland flow affecting the premises such that: - public benefit and enjoyment is maximised; - impacts on the asset life and integrity of park structures is minimised; - maintenance and replacement costs are minimised. | E107 Development for a Park(57) ensures works are provided in accordance with the requirements set out in Appendix B of the Planning scheme policy - Integrated design. |  |  |
| Riparian and wetland setbacks |  |  |  |
| PO108 Development provides and maintains a suitable setback from waterways and wetlands that protects natural and environmental values. This is achieved by recognising and responding to the following matters: - impact on fauna habitats; - impact on wildlife corridors and connectivity; - impact on stream integrity; - impact of opportunities for revegetation and rehabilitation planting; - edge effects. | E108 Development does not occur within: - 50m from top of bank for W1 waterway and drainage line - 30m from top of bank for W2 waterway and drainage line - 20m from top of bank for W3 waterway and drainage line - 100m from the edge of a Ramsar wetland, 50m from all other wetlands. Note - W1, W2 and W3 waterway and drainage lines, and wetlands are mapped on Schedule 2, Section 2.5 Overlay Maps – Riparian and wetland setbacks.. |  |  |
| Scenic amenity - Regionally significant (Hills) and Locally important (Coast) (refer Overlay map - Scenic amenity to determine if the following assessment criteria apply) |  |  |  |
| PO109 Landscaping - complements the coastal landscape character and amenity; - has known resilience and robustness in the coastal environment; Fences and walls: - do not appear visually dominant or conspicuous within its setting; - reduce visual appearance through the use of built form articulation, setbacks, and plant screening; - use materials and colours that are complementary to the coastal environment. Building design responds to the bayside location and complements the particular bayside character and amenity by adopting and incorporating a range of architectural character elements. Vegetation that contributes to bayside character and identity are: - retained; - protected from development diminishing their significance. | E109 Where located in the Locally Important (Coast) scenic amenity overlay: - landscaping comprises indigenous coastal species; - fences and walls are no higher than 1m; and - existing pine trees, palm trees, mature fig and cotton trees are retained. - where over 12m in height, the building design includes the following architectural character elements: - curving balcony edges and walls, strong vertical blades and wall planes; - balcony roofs, wall articulation expressed with different colours, curves in plan and section, and window awnings; - roof top outlooks, tensile structures as shading devices; - lightweight structures use white frame elements in steel and timber, bold colour contrast. |  |  |
| Transport noise corridors (refer Overlay map - Transport noise corridors to determine if the following assessment criteria apply) Note - This is for information purposes only. No requirements for accepted development or criteria for assessable development apply. Development located within a Transport Noise Corridor must satisfy the requirements of the Queensland Development Code. |

| Table 6.2.6.4.3 Setbacks |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Residential uses |  |  |  |  |  |  |  |  |  |  |
| Height of wall | Frontage primary |  |  | Frontage secondary to street |  |  | Frontage secondary to lane | Side non-built to boundary wall To OMP and wall | Rear To OMP and wall | Canal To OMP and wall |
|  | To wall | To OMP | To covered car parking space | To wall | To OMP | To covered car parking space | To OMP, wall and covered car parking space |  |  |  |
| Less than 4.5m | Min 1m | Min 1m | Min 5.4m | Min 1m | Min 1m | Min 5.4m | Min 0.5m | Min 1.5m | Min 1.5m | Min 4.5m |
| 4.5 to 8.5m | Min 1m | Min 1m | N/A | Min 1m | Min 1m | N/A | Min 0.5m | Min 2m | Min 2m | Min 4.5m |
| Greater than 8.5m | Min 5m | Min 3m | N/A | Min 2m | Min 1m | N/A | Min 0.5m | Min 2m up to 8.5m in height; plus 0.5m for every 3m in height or part thereof over 8.5m | Min 5m | Min 4.5m |

| Table 6.2.6.4.4 Built to boundary walls (Residential uses) |  |  |
| --- | --- | --- |
| Lot frontage width | Mandatory / Optional | Length and height of built to boundary wall |
|  |  | Urban neighbourhood precinct |
| Less than 7.5m | Mandatory - both sides unless a corner lot | Max Length: 80% of the length of the boundary Max Height: 8.5m |
| 7.5m to 12.5m | Mandatory - one side | Max Length: 70% of the length of the boundary Max Height: 10.5m |
| >12.5m to 18m | Optional: - on 1 boundary only; - where the built to boundary wall adjoins a lot with a frontage less than 18m. | Max Length: the lesser of 15m or 60% of the length of the boundary Max Height: 10.5m |
| Greater than 18m | As per QDC |  |

| Table 6.2.6.4.5 Car parking spaces |  |  |  |
| --- | --- | --- | --- |
| Site proximity | Land use | Maximum number of car spaces to be provided | Minimum number of car Spaces to be provided |
| Within 800m walkable Catchment* of a higher order centre | Non-residential | 1 per 30m2 GFA | 1 per 50m2 GFA |
|  | Residential – permanent/long term | N/A | 1 per dwelling |
|  | Residential – serviced/short term | 3 per 4 dwellings + staff spaces | 1 per 5 dwellings + staff spaces |
| Other (Wider catchment) | Non-residential | 1 per 20m2 GFA | 1 per 30m2 GFA |
|  | Residential – permanent/long term | N/A | 1 per dwelling |
|  | Residential – serviced/short term | 1 per dwelling + staff spaces | 1 per 5 dwellings + staff spaces |

Note - Car parking rates are to be rounded up to the nearest whole number. Note - Allocation of car parking spaces to dwellings is at the discretion of the developer. Note - Residential - Permanent/long term includes: Multiple dwelling(49), Relocatable home park(62), Residential care facility(65), Retirement facility(67). Note - Residential - Services/short term includes: Rooming accommodation(69) or Short-term accommodation(77).

Density Figures

Figure 6.2.6.4.1 - Kallangur

Figure 6.2.6.4.2 - Mango Hill

Figure 6.2.6.4.3 - Mango Hill East

Figure 6.2.6.4.4 - Murrumba Downs

Figure 6.2.6.4.5 Kippa-Ring

Movement network figures

Figure 6.2.6.4.6 - Dakabin

Figure 6.2.6.4.7 - Kallangur

Figure 6.2.6.4.8 - Mango Hill

Figure 6.2.6.4.9 - Mango Hill East

Figure 6.2.6.4.10 - Murrumba Downs

Figure 6.2.6.4.11 - Narangba east

Figure 6.2.6.4.12 - Petrie