Table 9.4.1.6.4.1 Requirements for accepted development - General residential zone - Urban neighbourhood precinct

| Requirements for accepted development |  | E Compliance - Yes - No | Council confirmation |
| --- | --- | --- | --- |
| General requirements |  |  |  |
| Boundary realignment |  |  |  |
| RAD1 | Lots created by boundary realignment: - contain all service connections to water, sewer, electricity and other infrastructure wholly within the lot they serve; - have constructed road access; - do not require additional infrastructure connections or modification to existing connections. - do not result in the creation of any additional lots; |  |  |
| RAD2 | Boundary realignment does not result in existing land uses on-site becoming non-complying with planning scheme requirements. Note - Examples may include but are not limited to: - minimum lot size requirements; - minimum or maximum required setbacks - parking and access requirements; - servicing and Infrastructure requirements; - dependant elements of an existing or approved land use being separately titled, including but not limited to: - Where premises are approved as Multiple dwelling(49) with a communal open space area, the communal open space cannot be separately titled as it is required by the Multiple dwelling(49) approval. - Where a commercial or industrial land use contains an ancillary office(53), the office(53) cannot be separately titled as it is considered part of the commercial or industrial use. - Where a Dwelling house(22) includes a secondary dwelling or associated outbuildings, they cannot be separately titled as they are dependent on the Dwelling house(22) use.. |  |  |
| RAD3 | Resulting lots comply with the following minimum lot sizes and dimensions: Zone (Precinct); Area; Frontage; Depth. General residential - Urban neighbourhood precinct; -; 32 m; 25 m. |  |  |
| RAD4 | Boundary realignment in the precinct does not result in more than 4 adjoining lots of the same lot type, as defined in ‘Table 9.4.1.6.4.3: Lot Types’ - Lot Types. |  |  |
| RAD5 | Boundary realignment does not result in the creation of additional building development opportunity within an area subject to an overlay map. |  |  |
| RAD6 | No new boundaries are located within 2m of High Value Areas as identified in Overlay map - Environmental areas. |  |  |
| RAD7 | Boundary realignment does not result in the clearing of any Habitat trees. |  |  |

Table 9.4.1.6.4.3: Lot Types

Density Figures

Figure 1 - Kallangur

Figure 2 - Mango Hill

Figure 3 - Mango Hill East

Figure 4 - Murrumba Downs

Figure 5 Kippa-Ring

Movement Figures

Figure 6 - Dakabin

Figure 7 - Kallangur

Figure 8 - Mango Hill

Figure 9 - Mango Hill East

Figure 10 - Murrumba Downs

Figure 11 - Narangba East

Figure 12 - Petrie